# SIMATS ENGINEERING

ASSESSMENT TOOL 1

# Experiments

NAME: K N RITHEESH

REG NO: 192572097

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0716

## COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and DNS under varying conditions

Assessment Tool Weightage: 40%

SOFTWARE: CISCO PACKET TRACER, WIRESHARK

EXPERIMENTS

QUESTIONS:4	Total Marks:40

- Design a simple LAN consisting of two PCs connected through a switch using Cisco Packet Tracer. Configure appropriate IP addresses for both hosts and verify connectivity using the ping command. Simultaneously capture the packet exchange using Wireshark. Identify the Ethernet frame fields such as Source MAC Address, Destination MAC Address, EtherType, and Frame Check Sequence (FCS). Explain how the Data Link layer uses MAC addressing to ensure successful frame delivery within the local network.

- Create a network topology consisting of two LANs connected through a router in Cisco Packet Tracer. Configure IPv4 addressing, subnet masks, and default gateways for all devices, and verify end-to-end communication using the ping command. Capture the ICMP packets using Wireshark and analyze the IPv4 header fields, including Source IP Address, Destination IP Address, Time-To-Live (TTL), Protocol Number, and Header Length. Explain the role of the Network layer in forwarding packets between different networks.

- Construct a local area network with multiple hosts connected through a switch in Cisco Packet Tracer. Clear the ARP cache on one of the hosts and initiate communication with another host using the ping command. Capture the communication in Wireshark and identify the ARP Request and ARP Reply packets. Analyze the sender and target MAC/IP addresses and explain how the Address Resolution Protocol resolves logical IP addresses into physical MAC addresses before data transmission.

- Develop a simple network consisting of two PCs connected through a router in Cisco Packet Tracer. Configure the required IP addresses and test connectivity using the ping command. Capture the ICMP traffic using Wireshark and identify the Echo Request and Echo Reply messages. Examine important ICMP fields such as Type, Code, Identifier, and Sequence Number, and calculate the Round Trip Time (RTT). Discuss how ICMP assists in network troubleshooting and connectivity verification.

- Create a client-server network in Cisco Packet Tracer by configuring a web server and a client PC. Enable the HTTP service on the server and access the hosted webpage from the client using a web browser. Capture the communication in Wireshark and identify the TCP three-way handshake, followed by the HTTP GET request and HTTP response messages. Analyze the application-layer communication and explain how TCP provides reliable delivery for HTTP traffic through connection establishment, acknowledgments, and ordered data transfer.

## RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficien t | Level 2 – Developing | Level 1 – Beginnin g |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologi es | Good understandin g with proper integration of most concepts | Basic understandin g; limited or partial integration | Poor understandin g; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementati on with minor issues | Basic implementati on with errors or inefficiencies | Incorrect or incomplete implementati on |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriat e use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentati on | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentatio n with necessary details | Basic documentatio n with missing details | Poor or incomplete documentatio n |

### Simple LAN with Two PCs Connected Through a Switch Aim

To design a simple LAN using Cisco Packet Tracer, configure IP addresses, verify connectivity using the ping command, capture packets using Wireshark, and analyze Ethernet frame fields.

### Network Topology

PC0 -------- Switch	PC1

### IP Configuration

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.1 | 255.255.255.0 |
| PC1 | 192.168.1.2 | 255.255.255.0 |

### Steps

  - Open Cisco Packet Tracer.

  - Place one switch and two PCs.

  - Connect both PCs to the switch using Copper Straight-Through cable.

  - Configure IP addresses on both PCs.

  - Open Command Prompt on PC0 and type: ping 192.168.1.2

  - Open Wireshark and start packet capture.

  - Observe Ethernet frames.

### Ethernet Frame Fields

| Field | Example | Purpose |
| --- | --- | --- |
| Destination MAC | 00:11:22:33:44:55 | Receiver's MAC address |
| Source MAC | AA:BB:CC:DD:EE:FF | Sender's MAC address |
| EtherType | 0x0800 | Indicates IPv4 packet |
| Payload | ICMP Data | Actual transmitted data |
| FCS | CRC Value | Detects transmission errors |

### Result

    - Ping is successful.

    - Ethernet frames are captured.

### Explanation

The Data Link Layer (Layer 2) uses MAC addresses to uniquely identify devices within the same LAN. Before transmitting data, the source device places the destination MAC address into the Ethernet frame. The switch forwards the frame only to the intended device based on its MAC address table. The Frame Check Sequence (FCS) ensures error detection during transmission.

### Two LANs Connected Through a Router Aim

To configure two LANs connected through a router, verify communication, and analyze IPv4 packet headers.

### Network Topology

PC0 ---- Switch ---- Router ---- Switch	PC1

### IP Configuration

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC0 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| Router G0/0 | 192.168.1.1 | 255.255.255.0 | - |
| Router G0/1 | 192.168.2.1 | 255.255.255.0 | - |
| PC1 | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 |

### Router Configuration

enable

configure terminal interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown einterface g0/1

ip address 192.168.2.1 255.255.255.0

no shutdown end Verification

ping 192.168.2.2

### IPv4 Header Fields

| Field | Example | Purpose |
| --- | --- | --- |
| Source IP | 192.168.1.2 | Sender IP |
| Destination IP | 192.168.2.2 | Receiver IP |
| TTL | 128 | Prevents routing loops |
| Protocol | 1 | Indicates ICMP |
| Header Length | 20 Bytes | Size of IPv4 header |

### Result

- Ping successful between different networks.

### Explanation

The Network Layer (Layer 3) forwards packets between different networks using IP addresses. Routers examine the destination IP address and forward packets to the correct network while decrementing the TTL value at every hop.

### ARP Packet Analysis Aim

To observe ARP Request and ARP Reply packets using Wireshark.

### Network Topology

PC0 ------- Switch	PC1

### IP Configuration

| Device | IP Address |
| --- | --- |
| PC0 | 192.168.1.10 |
| PC1 | 192.168.1.20 |

### Procedure

  - Configure both PCs.

  - Clear ARP cache: arp -d

  - Ping PC1 from PC0.

  - Capture packets in Wireshark.

### ARP Analysis ARP Request

| Field | Value |
| --- | --- |
| Sender MAC | PC0 MAC |
| Sender IP | 192.168.1.10 |
| Target MAC | 00:00:00:00:00:00 |
| Target IP | 192.168.1.20 |

### ARP Reply

| Field | Value |
| --- | --- |
| Sender MAC | PC1 MAC |
| Sender IP | 192.168.1.20 |
| Target MAC | PC0 MAC |
| Target IP | 192.168.1.10 |

### Result

    - ARP Request and Reply successfully captured.

### Explanation

### RP resolves a device's IP address into its physical MAC address before communication begins. The sender broadcasts an ARP Request, and the destination responds with its MAC address. The sender stores this information in its ARP cache for future communication.

### 4.ICMP Packet Analysis Aim

To capture ICMP Echo Request and Echo Reply messages and calculate RTT.

### Network Topology

PC0 ----- Router	PC1

### IP Configuration

| Device | IP Address |
| --- | --- |
| PC0 | 192.168.1.2 |
| Router | 192.168.1.1 / 192.168.2.1 |
| PC1 | 192.168.2.2 |

### Procedure

  - Configure IP addresses.

  - Ping PC1 from PC0.

  - Capture packets in Wireshark.

### ICMP Fields

| Field | Example | Purpose |
| --- | --- | --- |
| Type | 8 | Echo Request |
| Code | 0 | No Error |
| Identifier | 0x0001 | Identifies request |
| Sequence Number | 1 | Tracks packet order |

Echo Reply:

| Field | Value |
| --- | --- |
| Type | 0 |
| Code | 0 |

### Round Trip Time

RTT = Reply Time − Request Time Example:

Request = 0.002 s Reply = 0.005 s RTT = 3 ms

### Result

- ICMP Echo Request and Reply observed.

### Explanation

ICMP is used for network diagnostics. The ping command sends Echo Requests and receives Echo Replies to verify connectivity and measure network latency (RTT).

### 5.Client-Server Network (HTTP) Aim

To configure a web server and client, access a webpage, and analyze TCP and HTTP communication.

### Network Topology

Client PC ----- Switch	Web Server

### IP Configuration

| Device | IP Address |
| --- | --- |
| Client PC | 192.168.1.2 |
| Web Server | 192.168.1.10 |

### Configuration

  - Configure IP addresses.

  - Enable HTTP Service on the server.

  - Open Web Browser on the client.

  - Enter:

http://192.168.1.10

  - Start Wireshark capture.

### TCP Three-Way Handshake

| Step | Message |
| --- | --- |
| 1 | SYN |
| 2 | SYN-ACK |
| 3 | ACK |

### HTTP Communication

| Request | Response |
| --- | --- |
| HTTP GET | HTTP/1.1 200 OK |

### TCP Reliability

    - Connection Establishment: Uses the three-way handshake (SYN, SYN-ACK, ACK).

    - Acknowledgments: Every received segment is acknowledged to ensure delivery.

    - Ordered Delivery: Sequence numbers keep data in the correct order.

    - Retransmission: Lost packets are resent automatically.

### Result

    - The webpage loads successfully.

    - Wireshark captures the TCP handshake, HTTP GET request, and HTTP response.

### Explanation

The Application Layer uses HTTP to exchange web pages between the client and server. The Transport Layer uses TCP to provide reliable communication through connection establishment, acknowledgments, retransmission of lost packets, and ordered delivery of data.