SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0716

NAME: RITHEESH K N

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

1. A university network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. Lab A is assigned a /26 subnet, Lab B is assigned a /27 subnet, and Lab C is assigned a /28 subnet. Based on the given subnet masks, apply subnetting concepts to determine the network address of each lab. Further, identify the valid range of host IP addresses and the broadcast address for each subnet. Finally, calculate the total number of usable host IP addresses available in each lab’s subnet.
Given

- Network Address = 192.168.20.0/24

- Lab A → /26

- Lab B → /27

- Lab C → /28

Formula:

- Number of Hosts = 232-prefix

- Usable Hosts = 232-prefix-2

## Solution

### Lab A (/26)

Subnet Mask

255.255.255.192

Block Size

64

Network Address

192.168.20.0

Host Range

192.168.20.1 – 192.168.20.62

Broadcast Address

192.168.20.63

Usable Hosts

26-2=64-2=62

Usable Hosts = 62

### Lab B (/27)

Next available subnet begins from 192.168.20.64

Subnet Mask

255.255.255.224

Network Address

192.168.20.64

Host Range

192.168.20.65 – 192.168.20.94

Broadcast Address

192.168.20.95

Usable Hosts

25-2=32-2=30

Usable Hosts = 30

### Lab C (/28)

Next available subnet begins from 192.168.20.96

Subnet Mask

255.255.255.240

Network Address

192.168.20.96

Host Range

192.168.20.97 – 192.168.20.110

Broadcast Address

192.168.20.111

Usable Hosts

24-2=16-2=14

Usable Hosts = 14

## Final Result

| Lab | Prefix | Network Address | Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| Lab A | /26 | 192.168.20.0 | 192.168.20.1 – 192.168.20.62 | 192.168.20.63 | 62 |
| Lab B | /27 | 192.168.20.64 | 192.168.20.65 – 192.168.20.94 | 192.168.20.95 | 30 |
| Lab C | /28 | 192.168.20.96 | 192.168.20.97 – 192.168.20.110 | 192.168.20.111 | 14 |

2. A network administrator has configured two systems with the following details: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System 2 has IP address 192.168.1.190 with subnet mask 255.255.255.192. Using the given configuration, apply subnetting techniques to determine the subnet range, network address, and broadcast address for each system. Identify the valid host IP range for both subnets based on the given subnet mask. Calculate whether each IP address falls within its respective subnet range. Also, determine the number of usable hosts addresses available in each subnet.

Solution

Subnet block size

256-192=64

Subnets

0–63

64–127

128–191

192–255

### System 1

IP

192.168.1.130

Falls inside

128–191

Network Address

192.168.1.128

Broadcast Address

192.168.1.191

Host Range

192.168.1.129 – 192.168.1.190

Usable Hosts

64-2=62

Hence,

192.168.1.130 belongs to this subnet.

### System 2

IP

192.168.1.190

Falls inside

128–191

Network Address

192.168.1.128

Broadcast Address

192.168.1.191

Host Range

192.168.1.129 – 192.168.1.190

Usable Hosts

62

Hence,

192.168.1.190 also belongs to the same subnet.

## Final Result

| System | Network Address | Host Range | Broadcast Address | Valid IP? | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| System 1 | 192.168.1.128 | 192.168.1.129 – 192.168.1.190 | 192.168.1.191 | Yes | 62 |
| System 2 | 192.168.1.128 | 192.168.1.129 – 192.168.1.190 | 192.168.1.191 | Yes | 62 |

### Interpretation

Both systems belong to the same subnet because they share the same network address (192.168.1.128/26). Therefore, they can communicate directly without requiring a router.

3. An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for each department as follows: HR → /25, Sales → /26, IT → /27, and Admin → /28. Using the given subnet masks, apply subnetting techniques to assign suitable subnet addresses for each department. Determine the network address, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate the number of usable host addresses in each department. Finally, identify the remaining unused IP address range within the given network block.

## Solution

### HR (/25)

Network

172.16.0.0

Host Range

172.16.0.1 – 172.16.0.126

Broadcast

172.16.0.127

Usable Hosts

126

### Sales (/26)

Network

172.16.0.128

Host Range

172.16.0.129 – 172.16.0.190

Broadcast

172.16.0.191

Usable Hosts

62

### IT (/27)

Network

172.16.0.192

Host Range

172.16.0.193 – 172.16.0.222

Broadcast

172.16.0.223

Usable Hosts

30

### Admin (/28)

Network

172.16.0.224

Host Range

172.16.0.225 – 172.16.0.238

Broadcast

172.16.0.239

Usable Hosts

14

### Remaining Unused Range

After Admin subnet,

Remaining addresses begin from

172.16.0.240

Remaining range

172.16.0.240 – 172.16.255.255

## Final Result

| Department | Prefix | Network Address | Host Range | Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| HR | /25 | 172.16.0.0 | 172.16.0.1 – 172.16.0.126 | 172.16.0.127 | 126 |
| Sales | /26 | 172.16.0.128 | 172.16.0.129 – 172.16.0.190 | 172.16.0.191 | 62 |
| IT | /27 | 172.16.0.192 | 172.16.0.193 – 172.16.0.222 | 172.16.0.223 | 30 |
| Admin | /28 | 172.16.0.224 | 172.16.0.225 – 172.16.0.238 | 172.16.0.239 | 14 |

4. A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

### Given

Network

172.16.10.0/24

Subnet Mask

/27

IP Address

172.16.10.78

Second IP

172.16.10.95

## Method Used

Block Size

256-224=32

Subnets

0

32

64

96

128

160

192

224

## Solution

### Total Number of Subnets

Borrowed bits

27−24=3

23=8

Total Subnets

8

### Usable Hosts

25-2=30

Usable Hosts

30

### Subnet of 172.16.10.78

78 lies between

64–95

Network Address

172.16.10.64

Host Range

172.16.10.65 – 172.16.10.94

Broadcast

172.16.10.95

### heck IP 172.16.10.95

172.16.10.95 is

Broadcast Address

Therefore,It does not represent a valid host.

## Final Result

| Item | Value |
| --- | --- |
| Total Subnets | 8 |
| Usable Hosts/Subnet | 30 |
| Network Address | 172.16.10.64 |
| Host Range | 172.16.10.65 – 172.16.10.94 |
| Broadcast Address | 172.16.10.95 |
| Is 172.16.10.95 in same subnet? | Yes, but it is the broadcast address (not a usable host). |

### Interpretation

The IP address 172.16.10.78 belongs to subnet 172.16.10.64/27. The address 172.16.10.95 is the broadcast address of the same subnet and therefore cannot be assigned to any host.

5. A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

## Solution

### Compressed Address

2001:db8::ff00:42:8329

### Expanded Address

2001:0db8:0000:0000:0000:ff00:0042:8329

### Network Prefix (/64)

First 64 bits represent the network.

Network Prefix

2001:0db8:0000:0000::/64

### Host Portion

Remaining

64 bits

### Possible Host Addresses

264=18,446,744,073,709,551,616

Possible Host Addresses

18,446,744,073,709,551,616

## Final Result

| Item | Value |
| --- | --- |
| Original Address | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Compressed Address | 2001:db8::ff00:42:8329 |
| Expanded Address | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Network Prefix | 2001:0db8:0000:0000::/64 |
| Host Bits | 64 |
| Total Host Addresses | 18,446,744,073,709,551,616 |

.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |