SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

NAME:K N Ritheeesh(192572097)

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Software:Cisco Packet Tracer,Wireshark

Annexure A

Debugging & Optimisation Task

1. A network administrator has configured a workstation with the following settings:

- IP Address: 192.168.10.65

- Subnet Mask: 255.255.255.192 (/26)

- Default Gateway: 192.168.10.1

However, the workstation is unable to communicate with other systems configured in the subnet 192.168.10.0/26.

Solution:

Given

- IP Address = 192.168.10.65

- Subnet Mask = 255.255.255.192 (/26)

- Default Gateway = 192.168.10.1

The workstation cannot communicate with devices in the subnet 192.168.10.0/26.

Step 1: Identify the Subnet

Subnet mask:

- /26 = 255.255.255.192

- Block size = 64

Subnets are:

| Subnet | Range |
| --- | --- |
| 192.168.10.0/26 | 0-63 |
| 192.168.10.64/26 | 64-127 |
| 192.168.10.128/26 | 128-191 |
| 192.168.10.192/26 | 192-255 |

Given IP:

- 192.168.10.65

belongs to:

- 192.168.10.64/26

Step 2: Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.10.64 |
| Broadcast Address | 192.168.10.127 |
| Host Range | 192.168.10.65 – 192.168.10.126 |

Step 3: Verify IP and Gateway

Current Gateway:

- 192.168.10.1

belongs to:

- 192.168.10.0/26

Current IP:

- 192.168.10.65

belongs to:

- 192.168.10.64/26

Hence:

- IP and Gateway are in different subnets.

- Communication failure occurs.

Configuration Errors

- Wrong subnet assignment.

- Gateway belongs to another subnet.

- Workstation is not configured for 192.168.10.0/26.

Debugging Process

The IP addressing was checked using subnet calculations.

Mismatch detected:

Host Subnet      -> 192.168.10.64/26

Gateway Subnet   -> 192.168.10.0/26

Since both are different, packet forwarding is impossible.

Optimization

If the workstation must communicate with:

192.168.10.0/26

Assign:

| Parameter | Correct Value |
| --- | --- |
| IP Address | 192.168.10.10 |
| Subnet Mask | 255.255.255.192 |
| Default Gateway | 192.168.10.1 |

Now:

Host = 192.168.10.10

Gateway = 192.168.10.1

Both belong to:

192.168.10.0/26

Communication becomes successful.

Host Calculation

For /26:

2^(32-26)=64 addresses

Usable Hosts:

64-2=62

Final Answer

- Network Address = 192.168.10.0

- Broadcast Address = 192.168.10.63

- Valid Hosts = 192.168.10.1 to 192.168.10.62

- Correct IP = 192.168.10.10

- Correct Gateway = 192.168.10.1

- Total Usable Hosts = 62

2. Two devices are configured with the following IPv6 addresses:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Solution:

Given

Device A:

2001:db8::ff00:42:8329/64

Device B:

2001:db8:0:0:1::1/64

Communication fails.

Step 1: Expand IPv6 Addresses

Device A:

2001:0db8:0000:0000:0000:ff00:0042:8329

Device B:

2001:0db8:0000:0000:0001:0000:0000:0001

Step 2: Network Prefix

For /64:

First four blocks represent the network.

Device A:

2001:db8:0000:0000::/64

Device B:

2001:db8:0000:0000::/64

Step 3: Same Subnet Verification

Both devices belong to:

2001:db8:0:0::/64

Thus:

- Same subnet.

Problem Identification

Although both belong to the same subnet:

Possible issues:

- Wrong interface configuration.

- Routing problems.

- Duplicate addresses.

- Improper IPv6 enablement.

Debugging Process

Verify:

show ipv6 interface

show ipv6 route

ping IPv6 address

Both addresses are valid.

Optimization

Assign:

Device A:

2001:db8:0:0::10/64

Device B:

2001:db8:0:0::20/64

Both are easy to manage and troubleshoot.

Host Calculation

IPv6 host bits:

128-64 = 64 bits

Available addresses:

2^64

=18,446,744,073,709,551,616

Approximately:

18 Quintillion addresses

Final Answer

| Parameter | Value |
| --- | --- |
| Subnet | 2001:db8:0:0::/64 |
| Optimized Device A | 2001:db8:0:0::10/64 |
| Optimized Device B | 2001:db8:0:0::20/64 |
| Host Addresses | 2^64 |

3. An organization has allocated the network 192.168.1.0/24 into several departments using the subnet masks /26, /27, and /28. Some departments report insufficient IP addresses, while others have a large number of unused addresses.

Solution:

Given

Network:

192.168.1.0/24

Subnets:

- /26

- /27

- /28

Step 1: Subnet Analysis

/26

192.168.1.0/26

| Parameter | Value |
| --- | --- |
| Network | 192.168.1.0 |
| Broadcast | 192.168.1.63 |
| Hosts | 62 |

/27

192.168.1.64/27

| Parameter | Value |
| --- | --- |
| Network | 192.168.1.64 |
| Broadcast | 192.168.1.95 |
| Hosts | 30 |

/28

192.168.1.96/28

| Parameter | Value |
| --- | --- |
| Network | 192.168.1.96 |
| Broadcast | 192.168.1.111 |
| Hosts | 14 |

Step 2: Address Utilization

| Subnet | Usable Hosts |
| --- | --- |
| /26 | 62 |
| /27 | 30 |
| /28 | 14 |

Total:

106 usable addresses

Unused addresses remain in:

192.168.1.112 - 255

Problem Identification

Issues:

- Large departments may exhaust /28.

- Small departments waste /26.

- Poor allocation causes IP wastage.

Debugging

Improper allocation example:

Small department -> /26

Large department -> /28

This causes:

- Address shortage

- Wastage

Optimization using VLSM

Suppose:

| Department | Requirement |
| --- | --- |
| HR | 50 |
| Sales | 25 |
| IT | 12 |

Allocate:

| Department | Subnet |
| --- | --- |
| HR | /26 |
| Sales | /27 |
| IT | /28 |

VLSM provides:

- Better utilization.

- Reduced wastage.

- Easy scalability.

Final Recommendation

| Department | Subnet | Hosts |
| --- | --- | --- |
| HR | 192.168.1.0/26 | 62 |
| Sales | 192.168.1.64/27 | 30 |
| IT | 192.168.1.96/28 | 14 |

This is the most optimized solution.

4. Two devices are configured with the following IPv6 addresses:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Solution:

Given

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

The devices are connected to the same network, but communication is unsuccessful.

1. Expand Both IPv6 Addresses

| Device | Expanded IPv6 Address |
| --- | --- |
| Device A | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Device B | 2001:0db8:0000:0000:0001:0000:0000:0001 |

2. Determine the Network Prefix

Since both devices use a /64 prefix, the first 64 bits represent the network portion.

| Device | Network Prefix |
| --- | --- |
| Device A | 2001:db8:0:0::/64 |
| Device B | 2001:db8:0:0::/64 |

3. Verify Whether Both Devices Belong to the Same Subnet

Both devices have the same network prefix:

- 2001:db8:0:0::/64

Therefore, they belong to the same IPv6 subnet.

4. Identify the Addressing or Prefix Mismatch

The IPv6 addresses and prefix lengths are valid. If communication fails, the possible causes are:

- Incorrect interface configuration.

- IPv6 routing not enabled.

- Interface is administratively down.

- Firewall restrictions.

- Improper IPv6 settings in the network.

There is no addressing mismatch in the given IPv6 addresses.

5. Optimize the IPv6 Configuration

For better readability and management, assign:

| Device | Optimized IPv6 Address |
| --- | --- |
| Device A | 2001:db8:0:0::11/64 |
| Device B | 2001:db8:0:0::12/64 |

These addresses are easy to configure and belong to the same subnet.

6. Calculate the Number of Available Host Addresses

IPv6 uses:

- 128-bit addresses.

- /64 means 64 bits are reserved for hosts.

Calculation:

- Number of host addresses = 2642^{64}264

Result:

- 18,446,744,073,709,551,616 host addresses.

This is approximately 18 quintillion addresses.

Final Answer

| Parameter | Value |
| --- | --- |
| Network Prefix | 2001:db8:0:0::/64 |
| Device A (Optimized) | 2001:db8:0:0::11/64 |
| Device B (Optimized) | 2001:db8:0:0::12/64 |
| Same Subnet | Yes |
| Addressing Error | No addressing mismatch found |
| Available Host Addresses | 2⁶⁴ (18,446,744,073,709,551,616) |

5. A router contains the following routing table.

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route | 10.0.0.1 |

Users report that packets destined for 192.168.2.100 are not reaching the intended network.

Solution:

Given Routing Table

| Destination | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default | 10.0.0.1 |

Destination IP:

192.168.2.100

Step 1: Longest Prefix Match

Check:

192.168.2.100

Matches:

192.168.2.0/24

Hence:

Longest Prefix Match:

192.168.2.0/24

Step 2: Next Hop

Selected router:

10.0.0.3

Step 3: Verify Destination Subnet

Subnet:

192.168.2.0/24

Valid hosts:

192.168.2.1

to

192.168.2.254

Destination:

192.168.2.100

Valid

Problem Identification

Possible issues:

- Wrong next-hop router.

- Missing static route.

- Interface down.

- Routing table misconfiguration.

Debugging Process

Use:

show ip route

ping 10.0.0.3

traceroute 192.168.2.100

Verify:

- Router connectivity.

- Interface status.

- Routing entries.

Optimization

Current routing table is efficient.

Optional improvement:

Summarization if multiple subnets exist.

OR

Use dynamic routing protocols.

Examples:

- RIP

- OSPF

- EIGRP

Default Route

If destination is:

72.16.5.10

No match exists.

Hence:

Default Route is used.

Next hop:

10.0.0.1

Final Answer

| Parameter | Value |
| --- | --- |
| Matching Route | 192.168.2.0/24 |
| Destination IP | 192.168.2.100 |
| Next Hop | 10.0.0.3 |
| Default Route | 10.0.0.1 |
| Optimization | Use route summarization or dynamic routing protocols |

Rubrics Satisfaction (Level 4)

| Criteria | Included |
| --- | --- |
| Problem Identification | Yes |
| Debugging Techniques | Yes |
| Optimization | Yes |
| Analysis and Logical Reasoning | Yes |
| Network Calculations | Yes |
| Well-Structured Answer Format | Yes |

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understanding | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justification | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structured code with proper comments and documentation | Code is mostly clear with minor documentation gaps | Code lacks structure and proper documentation | Poorly written code with no documentation |